Surface Tension Standard Operating Procedure

- Download and Extract Surface Tension .zip file. If not installed already, install the following MATLAB Add-ons (these may be founding using the Add-on explorer):

  - Image Acquisition Toolbox

  - Image Processing Toolbox

  - MATLAB Support Package for USB Webcams

  - Image Acquisition Toolbox Support Package for OS Generic Video Interface

- Place Surface Tension test apparatus on a flat surface. Power backlight with a micro-USB and connect the USB camera to your laptop.

- Run “imageCaptureGUI.m”. The image capture interface will open, revealing a live feed of the camera. Dials on the camera stand move the camera up and down, and towards and away from the needle. Zoom, focus and aperture may be adjusted on the camera lens. Ensure that the needle is focused on the center of the camera display.

- Input material name, needle gauge and the total number of images to take in the burst (80-120 images is recommended).

- Once ready to sample data, select “Start Capture” and slowly drop droplets from the needle during the camera capture. 3-4 different drops while the camera is capturing is ideal.

- Once the camera has captured droplet images, photos taken will show on the left side of the interface. Select 4-5 droplet images that appear "good". A “good” droplet image is given below:

- Run “pendant_drop_analysis.m”. Navigate to the correct material and needle gauge folders and select the “Images” folder to have the script analyze the droplet photos.

- Input the density of your material, as well as the surrounding air density (Boulder, Co is default.)

- Select the needle meniscus location, and the left and right sides of the needle. Input the width of the needle (outer diameter).

- After the script runs, the average surface tension of selected droplet photos is outputted to the user.